Experiment 8: Interrupt-Driven Systems with

Semaphores

This experiment explores the integration of

hardware interrupts

and

semaphores

in embedded real-time systems. You

will learn how interrupts signal events from hardware to software, and how semaphores coordinate safe data sharing

between an Interrupt Service Routine (ISR) and the main application thread — a foundational pattern in RTOS-based

embedded design.

EMBEDDED SYSTEMS LAB

EXPERIMENT 8

Aim

The objective of this experiment is to design and implement an interrupt-driven embedded system that uses a

binary

semaphore

to synchronise an ISR with the main program loop. By the end of this experiment, you should be able to:

1

Understand Interrupts

Explain how hardware interrupts work and how an ISR

is triggered and executed on an embedded processor.

2

Apply Semaphores

Use a binary semaphore to safely pass event signals

from an ISR to the main task without data corruption.

3

Implement in Code

Write and test interrupt + semaphore code on a target

microcontroller using an RTOS or bare-metal

approach.

4

Observe Behaviour

Record and analyse timing, latency, and correctness of

event handling under real hardware conditions.

The Interrupt Concept

An

interrupt

is a hardware or software signal that temporarily pauses the CPU's current execution and redirects it to a

dedicated handler called the

Interrupt Service Routine (ISR)

. This allows the processor to respond to real-world events

— such as a button press, timer overflow, or UART data arrival — immediately and without continuous polling.

Without Interrupts (Polling)

The CPU continuously checks peripheral status registers

in a loop. This wastes CPU cycles and increases

response latency, especially when events are infrequent.

With Interrupts

The CPU executes its main task freely. When an event

occurs, hardware asserts an interrupt line, the CPU saves

its state, executes the ISR, then resumes — achieving

near-instant response.

ISR Limitations

While ISRs enable fast hardware response, they operate under strict constraints. Violating these rules leads to

system

crashes, race conditions, and unpredictable behaviour

. Every embedded engineer must internalise these limitations.

Must Be Short

ISRs should execute in microseconds. Long ISRs block

other interrupts and degrade system responsiveness.

No Blocking Calls

Never call blocking functions like

delay()

,

printf()

, or

RTOS

pend

inside an ISR — these can deadlock the

system.

Shared Data Risks

Variables shared between an ISR and main code must

be declared

volatile

and protected against concurrent

access.

Limited Stack

ISRs use the main stack. Deep nesting or large local

variables risk stack overflow on memory-constrained

devices.

The golden rule:

ISR signals the event; the main task handles the work.

Keep ISRs minimal and delegate

processing to the application layer.

The Role of Semaphores

A

semaphore

is a synchronisation primitive used to signal between concurrent execution contexts — in this case,

between an ISR and the main task. A

binary semaphore

acts like a flag: the ISR

g

i

v

e

s

(posts) it upon detecting an event,

and the main task

t

a

k

e

s

(pends) it to begin processing.

Main Task

Pends

ISR Posts

Semaphore

Hardware

Event

This pattern cleanly separates

detection

(ISR) from

processing

(main task), keeping the ISR short while allowing the

main loop to perform complex operations safely. In an RTOS environment, the semaphore

give

may also unblock a

waiting task, enabling priority-based scheduling.

Binary Semaphore

Value: 0 (unavailable) or 1 (available)

Single event signalling

ISR gives; task takes

Counting Semaphore

Value: 0 to N (tracks multiple events)

Queues multiple occurrences

Used when ISR fires repeatedly

System Design

The experiment system consists of a microcontroller (e.g., STM32 or Arduino) connected to an external push-button

(

interrupt source) and an LED (output indicator). The design follows a clean two-layer architecture separating interrupt

detection from application logic.

The

hardware layer

generates a GPIO falling-edge interrupt on button press. The

software layer

uses a binary

semaphore to bridge the ISR and the main application task, which toggles the LED as a visible confirmation of the

handled event.

How It Works — Step by Step

1

Initialisation

Configure the GPIO pin as input with interrupt on falling edge. Create and initialise the binary semaphore to 0

(

empty). Enable the NVIC interrupt channel.

2

Main Loop Blocks

The main task calls

semaphoreTake()

with a timeout or indefinite wait. It blocks here, consuming zero CPU

cycles until the semaphore is signalled.

3

Button Pressed

The user presses the button. Hardware detects the falling edge on the GPIO pin and raises the interrupt

request to the CPU.

4

ISR Executes

CPU saves context and jumps to the ISR. The ISR calls

semaphoreGiveFromISR()

to post the semaphore, then

returns immediately.

5

Task Unblocks

The semaphore give unblocks the main task. It takes the semaphore, toggles the LED, and loops back to block

again — ready for the next event.

Implementation

Below is a representative bare-metal C implementation for an STM32-class microcontroller using CMSIS and a simple

binary semaphore modelled with a

volatile

flag. In an RTOS context (e.g., FreeRTOS), replace the flag with

xSemaphoreGiveFromISR()

and

xSemaphoreTake()

.

/* --- Global semaphore flag --- */

volatile uint8_t semaphore =

0

;

/* --- GPIO Interrupt ISR --- */

void

EXTI0_IRQHandler

(

void)

{

if (EXTI->PR & EXTI_PR_PR0) {

EXTI->PR |= EXTI_PR_PR0;

/* Clear pending flag */

semaphore =

1

;

/* Give semaphore     */

}

}

/* --- Main Task --- */

int

main

(

void)

{

HAL_Init

()

;

GPIO_Init

;

()

/* Configure button (PA0) + LED (PA5) */

NVIC_EnableIRQ

EXTI0_IRQn);

(

while (

1

{

)

/* Take semaphore (spin-wait; use RTOS pend in production) */

if (semaphore ==

1

{

)

semaphore =

0

;

HAL_GPIO_TogglePin

(

GPIOA, GPIO_PIN_5);

/* Toggle LED */

}

}

}

In FreeRTOS, replace the

volatile

flag with

xSemaphoreHandle

and use

xSemaphoreGiveFromISR()

in the ISR and

xSemaphoreTake(sem, portMAX_DELAY)

in the task to enable true blocking without CPU spin.

Observations

During lab execution, record the following measurements and verify system behaviour against expected results. Use an

oscilloscope or logic analyser to capture interrupt latency and LED toggle timing.

Parameter

Expected

Observation Notes

Interrupt latency (button

→

ISR entry)

<

1 µs (hardware

)

Measure with logic analyser on ISR

GPIO toggle

Semaphore give

→

task unblock delay

)

<

10 µs (bare-metal

Time from ISR exit to LED toggle start

LED toggles per button press

Exactly 1

Verify no missed or double events

CPU load during idle (no button)

Spin: ~100% / RTOS: ~0%

Compare bare-metal vs. RTOS

implementation

Bounce events (mechanical button)

Multiple edges possible

Check if debounce logic is needed

If multiple LED toggles occur per press, the button is

bouncing

. Add a software debounce delay or hardware RC

filter (10 k

Ω

+ 100 nF) and re-observe.

Conclusion

This experiment demonstrated a core embedded systems pattern: using a

hardware interrupt to detect events

and a

binary semaphore to synchronise

the ISR with the main application task. This separation of concerns is critical to

building responsive, reliable, and maintainable embedded software.

🔌

Interrupts

Enable immediate hardware

response without CPU polling,

reducing latency and power

consumption significantly.

🚦

Semaphores

Provide a safe, structured

mechanism to signal between ISR

and task contexts without shared-

variable race conditions.

⚡

ISR Discipline

Keeping ISRs short and

delegating work to tasks is

essential for predictable timing in

real-time systems.

The interrupt + semaphore pattern is the backbone of event-driven embedded firmware — mastering it prepares you

for professional RTOS development on any platform.

EXPERIMENT COMPLETE

RTOS READY