Experiment 5: Super Loop Based Embedded

System

A practical exploration of bare-metal embedded programming using the super loop architecture — implementing

multiple concurrent tasks on a microcontroller without an RTOS.

EMBEDDED SYSTEMS

BARE-METAL PROGRAMMING

STM32 / HAL

Aim

Primary Objective

Implement a

super loop architecture

on an embedded

microcontroller platform using STM32 HAL libraries and

a bare-metal programming approach.

Secondary Objective

Perform

multiple tasks sequentially

within a single

infinite loop — including LED control, button polling,

and sensor reading — without any operating system

overhead.

This experiment demonstrates how real-world embedded systems can manage multiple peripherals using only

a structured main loop and timing counters.

Introduction

The

super loop

(also called a bare-metal loop or foreground/background system) is one of the most fundamental

architectures in embedded software design. At its core, it uses a single

while(1)

construct that runs indefinitely from

power-on until the system is reset.

Each iteration of the loop executes a set of tasks in sequence. Because the loop never exits, tasks are repeatedly

serviced — giving the illusion of simultaneous execution. This approach is ideal for

resource-constrained

microcontrollers

where the complexity and overhead of an RTOS are not justified.

Core Principle

Uses an infinite

while(1)

loop to execute tasks

repeatedly in a deterministic sequence.

Key Advantage

Minimal memory footprint — no kernel stack, no

scheduler, no context switching overhead.

Theory

Foreground — Interrupt Context

The

SysTick timer

fires at a regular interval (typically

every 1 ms). Its ISR increments the HAL tick counter,

providing a global time reference. This is the

"foreground" layer that services time-critical hardware

events asynchronously.

SysTick interrupt at 1 ms period

Increments

uwTick

counter

Serves as system heartbeat

Background — Main Loop

The

main loop

runs continuously in the background. It

reads the tick counter via

HAL_GetTick()

and compares

against stored timestamps to decide whether each task

is due for execution.

Polls

HAL_GetTick()

each iteration

Compares elapsed time per task

Executes task if interval has passed

HAL_GetTick()

returns the number of milliseconds elapsed since system startup, sourced from the SysTick-

driven counter — enabling non-blocking time management in the background loop.

Concept: Cooperative Task Scheduling

No Blocking Delays

Cooperative Tasks

No RTOS

The super loop concept is built on three pillars: operating without an RTOS kernel, using cooperative (non-preemptive)

task execution, and completely avoiding blocking delays. Each task checks the tick count and runs only when its time

slice arrives — then immediately returns control to the loop.

No RTOS

No scheduler, no task priorities, no

kernel — just bare hardware and your

code.

Cooperative

Tasks voluntarily yield after

completion. No preemption occurs

between tasks.

Non-Blocking

Replace

HAL_Delay()

with tick

comparison — the loop never stalls.

System Tasks

Three peripheral tasks are managed within the super loop, each with its own timing requirement and hardware interface.

1

LED Blinking

Toggles an onboard or external

LED at a defined interval.

Validates that the timing

mechanism is functioning

correctly — the most basic system

health indicator.

2

Button Reading

Polls the state of a digital push-

button input. Polled frequently to

capture brief presses reliably.

State changes can trigger actions

or log events.

3

IR Sensor Reading

Reads the output of an infrared

proximity or distance sensor.

Sampled at a moderate rate

sufficient for detecting

environmental changes in real

time.

Working Principle

Software Counter-Based Timing

Each task maintains its own

last-execution timestamp

stored

in a

uint32_t

variable. On every loop iteration, the current tick

value is fetched via

HAL_GetTick()

and compared against the

stored value. If the difference meets or exceeds the task's

interval, the task runs and the timestamp is updated.

This pattern ensures tasks are

time-multiplexed

without

blocking each other — the loop continues iterating freely

regardless of whether any task executed.

This approach handles

tick counter overflow

gracefully when using unsigned arithmetic —

subtraction wraps correctly after ~49.7 days at 1 ms

resolution.

Pseudo-Code Structure

uint32_t t_led =

0

;

uint32_t t_btn =

0

;

uint32_t t_ir =

0

;

while

(

1

)

{

uint32_t now =

HAL_GetTick

;

()

if

(

now - t_led >=

500

)

{

LED_Toggle

()

;

t_led = now;

}

if

(

now - t_btn >=

50

)

{

Button_Read

()

;

t_btn = now;

}

if

(

now - t_ir >=

200

)

{

IR_Read

()

;

t_ir = now;

}

}

Implementation: Task Timing Configuration

Each peripheral task is assigned a fixed execution interval tuned to its functional requirements. Faster tasks consume

marginally more CPU cycles per second but remain negligible in a bare-metal loop.

ms

500

LED Blink

Toggle period — 1 Hz visible blink rate,

suitable for visual status indication.

ms

50

Button Poll

Hz sampling — fast enough to

20

reliably detect short button presses

without debounce loss.

200

ms

IR Sensor

5

Hz read rate — adequate for

detecting proximity changes in

typical test scenarios.

Task

Interval

Frequency

Priority

LED Toggle

500

ms

2

Hz

Low

Button Read

50

ms

20

Hz

High

IR Sensor Read

200

ms

5

Hz

Medium

Observations

LED Blinked

Correctly

The onboard LED toggled

at the expected 500 ms

interval throughout the

experiment. No missed

cycles or timing drift were

observed during the test

duration, confirming

accurate tick-based

scheduling.

Button Detected

Properly

Press and release events

were reliably captured at

the 50 ms polling rate.

Short presses as brief as

~80 ms were consistently

registered without

debounce-related false

triggers.

Sensor Values

Changed with

Distance

The IR sensor output varied

predictably as an object

was moved toward and

away from the sensor.

Values updated smoothly

every 200 ms, with no stale

readings observed in the

serial output.

Conclusion

Multitasking Without

RTOS

The super loop architecture

successfully demonstrates

cooperative multitasking. Multiple

peripherals were managed

concurrently using only a single

loop and tick-based counters.

Bare-Metal Efficiency

The system operates entirely

without OS overhead — no kernel,

no scheduler. This results in

deterministic, lightweight

execution ideal for resource-

constrained MCUs.

Suitable for Simple

Systems

For applications with a small, well-

defined task set and no hard real-

time constraints, the super loop

remains a practical, maintainable,

and highly reliable design choice.

Key Takeaway:

The super loop pattern is a foundational technique every embedded engineer should master

before advancing to RTOS-based designs. Its simplicity is its strength.