Ultrasonic Sensor Interfacing with STM32

EMBEDDED SYSTEMS LAB

STM32F446RE

A hands-on experiment to interface the HC-SR04 ultrasonic sensor with the STM32F446RE microcontroller, measure

real-time distance, and indicate proximity using an LED.

Aim

Objective

To interface an

HC-SR04 ultrasonic sensor

with the

STM32F446RE microcontroller and measure distance using

LED indication as a visual output mechanism.

Configure GPIO and Timer peripherals on STM32

Measure time-of-flight of ultrasonic pulses

Indicate proximity via LED ON/OFF logic

Apparatus Required

STM32 Nucleo-F446RE

ARM Cortex-M4 based development board with onboard

ST-LINK debugger and abundant GPIO peripherals.

HC-SR04 Sensor

Ultrasonic distance sensor with TRIG and ECHO pins,

measuring distances from 2 cm to 400 cm.

Jumper Wires & USB Cable

Male-to-female jumper wires for connections and a USB

cable for programming and power supply.

STM32CubeIDE & CubeMX

STMicroelectronics' official IDE and configuration tool for

project setup, code generation, and debugging.

Introduction

Ultrasonic sensors are non-contact distance measurement devices that operate on the principle of

sound wave

propagation

. They are widely used in robotics, automation, and safety systems for reliable proximity sensing.

Trigger Pulse

The microcontroller sends a

short 10 µs HIGH pulse on the

TRIG pin to initiate a

measurement cycle.

Echo Signal

The sensor emits 8 ultrasonic

bursts at 40 kHz and waits for the

reflected echo to return on the

ECHO pin.

Distance Calculation

The time elapsed between the

trigger and echo is used to

compute the distance to the

object in front of the sensor.

Theory

Time-of-Flight Principle

The HC-SR04 sensor uses the

time-of-flight (ToF)

technique. The TRIG pin is driven HIGH for 10 µs, causing

the sensor to emit an ultrasonic burst. The ECHO pin

goes HIGH for a duration proportional to the travel time

of sound.

Speed of sound in air

≈

m/s

343

at room

temperature (25°C).

Distance Formula

Since the sound travels

t

o

the object and

b

a

c

k

, the total

path is divided by 2:

d = (v ×

Δ

t) / 2

where v = speed of sound,

Δ

t = echo pulse width

TRIG Pin

Sends a 10 µs HIGH

pulse to start

measurement

ECHO Pin

Returns HIGH pulse

proportional to

distance

v = 343 m/s

Speed of sound used in

the formula

÷ 2 Factor

Accounts for round-

trip travel of sound

Working Principle

Measure

ECHO

Echo

Reflection

Ultrasonic

Burst

MCU

Trigger

The working cycle is deterministic and repeatable. The STM32 timer peripheral captures the rising and falling edges of

the ECHO pin, giving a precise pulse width in microseconds. This value is then converted to centimetres using the

distance formula.

System Design

Pin Configuration

TRIG

→

GPIO Output pin (e.g., PA5)

ECHO

→

Timer Input Capture pin (e.g., PA6)

LED

→

GPIO Output pin for indication

GND & VCC

→

5V power from Nucleo board

Peripheral Usage

TIM2 / TIM3

used for microsecond-level timing

Input Capture mode captures ECHO edges

Timer prescaler set to achieve 1 µs resolution

LED threshold set (e.g., object within 20 cm)

Implementation

01

GPIO Configuration

In STM32CubeMX, configure the TRIG pin as

GPIO Output

and the ECHO pin as

Timer Input Capture

. Set LED pin as

GPIO Output.

02

Timer Initialisation

Initialise a timer (e.g., TIM2) with a

1

MHz clock

(prescaler =

SystemClock/1MHz − 1). Enable Input Capture on the ECHO

channel.

03

Distance Calculation

Capture timestamps on the rising and falling edges of

ECHO. Compute

Δ

t

in microseconds and apply:

d

i

s

t

a

n

c

e

=

(

3

4

3

×

Δ

t

)

/

(

2

×

1

0

0

0

0

)

cm.

04

LED Indication Logic

Compare computed distance against a threshold (e.g., 20

cm). Turn

LED ON

if object is closer, turn

LED OFF

if farther

away.

Generate project code using STM32CubeMX and complete the distance logic inside the

while(1)

loop in

main.c

.

Observations

Results Recorded

Distance Range

LED State

Accuracy

0

– 20 cm

ON (Close)

cm

±1

21

– 100 cm

OFF (Far)

±2

cm

>

100 cm

OFF

±4

cm

Accurate distance readings were consistently observed during

testing. Minor errors were noted due to ambient temperature

variations and acoustic reflections from nearby surfaces.

Environmental factors such as temperature, humidity, and

surface texture can affect the speed of sound and introduce

measurement errors.

✅

LED ON

Triggered for objects closer than 20 cm

⚪

LED OFF

Remains off for objects beyond

threshold

⚠

Minor Errors

Due to temperature and surface

reflections

Conclusion

The experiment successfully demonstrated interfacing of the

HC-SR04 ultrasonic sensor

with the STM32F446RE

microcontroller. Real-time distance measurement was achieved with good accuracy using the

time-of-flight principle

and timer-based pulse capture.

Interfacing Achieved

Sensor successfully integrated with STM32 GPIO and

Timer peripherals

Time-of-Flight Used

Precise distance measured from echo pulse width via

hardware timer

Accurate Results

Timer provided high resolution timing at 1 µs granularity

Real-World Applications

Applicable in obstacle detection, parking sensors, and

robotic navigation