PWM Signal Generation using STM32

EMBEDDED SYSTEMS EXPERIMENT

A hands-on laboratory experiment demonstrating Pulse Width Modulation signal generation using the STM32F446RE

microcontroller, with LED brightness control through variable duty cycle configuration.

Aim

Objective

Generate a PWM signal using the

STM32F446RE

microcontroller and control LED brightness by varying the

duty cycle of the output signal.

Configure hardware timer for PWM output

Adjust duty cycle values in firmware

Observe corresponding LED brightness levels

Apparatus Required

The following hardware and software tools are required to complete this experiment successfully.

STM32 Nucleo-

F446RE

The primary microcontroller

development board

featuring the ARM Cortex-

M4 core with onboard ST-

LINK debugger.

USB Cable

Used for powering the

Nucleo board and

establishing a serial

communication link with

the host PC.

STM32CubeIDE

Eclipse-based integrated

development environment

for writing, compiling, and

flashing firmware to the

target device.

STM32CubeMX

Graphical initialisation tool

for configuring peripherals,

clock trees, and generating

initialisation code

automatically.

Introduction to PWM

Pulse Width Modulation (PWM) is a widely used technique in embedded systems that enables control of analog devices

through digital signals. By rapidly switching a digital output between HIGH and LOW states, PWM effectively simulates

varying voltage levels.

The core principle lies in varying the

ON time

and

OFF time

of a periodic signal. A longer ON period results in a higher

effective voltage at the output, while a shorter ON period reduces it — all while the signal frequency remains constant.

PWM is commonly used for motor speed control, LED dimming, servo positioning, and audio signal generation

in embedded applications.

Theory

Understanding the key parameters of a PWM signal is essential before implementation. These parameters determine the

behaviour and effectiveness of the output signal.

Duty Cycle

The percentage of time the signal remains HIGH

within one complete period. A

50

% duty cycle

means

the signal is ON for half the period and OFF for the

other half.

Frequency

The number of complete PWM cycles per second,

measured in Hz. Higher frequencies produce

smoother output but may impose constraints on

timer configuration.

Output Voltage

The effective (average) output voltage is directly

proportional to the duty cycle. A higher duty cycle

produces a higher average voltage at the output pin.

LED Brightness Relationship

Since LED brightness depends on average current, a

higher duty cycle

results in greater perceived

brightness, while lower duty cycles dim the LED

proportionally.

PWM Parameters

Four fundamental parameters define any PWM waveform. Together, they fully characterise the signal behaviour on the

STM32 timer output.

On the STM32,

Period

is set via the Auto-Reload Register (ARR), and

Duty Cycle

is controlled by the

Capture/Compare Register (CCR).

Working Principle

The STM32 hardware timer generates PWM by continuously comparing its internal counter value against a threshold

register. This comparison logic directly drives the output pin state.

Square

Wave

Output

LOW

Output

HIGH

Timer

Counts

The

ARR (Auto-Reload Register)

defines the period of the waveform, while the

CCR (Capture/Compare Register)

sets

the duty cycle threshold. Adjusting CCR dynamically changes the pulse width without affecting the overall frequency.

CNT < CCR → HIGH

The output pin is driven HIGH while the counter value is

below the CCR threshold, forming the ON portion of the

pulse.

CNT ≥ CCR → LOW

Once the counter reaches or exceeds CCR, the output is

driven LOW until the counter resets at ARR, completing

the cycle.

Implementation

The experiment uses

TIM2

on the STM32F446RE, configured in PWM Generation mode. STM32CubeMX is used to set up

the timer, and the generated code is modified to vary the CCR value for duty cycle control.

Configure TIM2 in STM32CubeMX

Select TIM2, set the channel to PWM Generation CH1, and

assign the output pin.

Set Prescaler and ARR

Calculate and enter Prescaler and Auto-Reload Register

values to achieve the desired PWM frequency.

Control Duty Cycle via CCR

In the generated code, modify the CCR value using

__HAL_TIM_SET_COMPARE()

to vary the duty cycle.

Observe LED Brightness

Flash the firmware and observe how LED brightness

changes proportionally as the CCR value is varied.

Conclusion

This experiment successfully demonstrated the generation of a PWM signal using the STM32F446RE and validated the

control of LED brightness through duty cycle variation.

✅

PWM Successfully

Generated

TIM2 was correctly configured to

produce a stable PWM output at

the desired frequency using

STM32CubeIDE and CubeMX.

💡

Smooth Brightness

Control

LED brightness varied smoothly

and proportionally across all

tested duty cycle values, from fully

off to fully on.

📐

Duty Cycle

Relationship Confirmed

Results clearly demonstrate the

linear relationship between duty

cycle percentage and effective

output voltage at the GPIO pin.

The experiment confirms that PWM is an effective and efficient method for analog-level control using digital

hardware — a foundational skill in embedded systems development.