Task Priority in FreeRTOS

A deep-dive into how FreeRTOS schedules tasks based on priority, how the preemptive scheduler selects which task

runs, and what happens when two tasks compete for CPU time. Designed for embedded systems engineers and

firmware developers ready to master real-time task management.

FREERTOS

RTOS INTERNALS

TASK SCHEDULING

Aim — Analyse Priority Effect

This document investigates how task priority assignments directly influence execution order, CPU allocation, and

system responsiveness in a FreeRTOS environment. By observing a controlled two-task system, we establish a clear

understanding of priority-driven scheduling behaviour.

Core Question

What happens when two tasks

with different priorities are

created and allowed to run

simultaneously?

Expected Insight

The higher-priority task should

always preempt or block the

lower-priority task from executing

until it yields or blocks.

Practical Value

Understanding this behaviour is

critical for designing

deterministic, real-time

embedded applications with tight

timing constraints.

Scheduler Basics

FreeRTOS uses a

tick-based preemptive scheduler

that runs at a configurable frequency (typically 1 kHz). At each tick

interrupt, the scheduler evaluates which task has the highest priority in the Ready state and switches context if

necessary.

Scheduler Modes

Preemptive:

Higher-priority tasks immediately

preempt lower ones

Co-operative:

Tasks yield voluntarily; no forced

context switch

Time-sliced:

Equal-priority tasks share CPU in

round-robin fashion

Key Scheduler Concepts

Tick Interrupt:

Heartbeat driving the scheduler

Context Switch:

Saving and restoring task register

state

Ready Queue:

Sorted list of tasks eligible to run

Idle Task:

Lowest-priority task, always ready to run

Priority-Based Execution

In FreeRTOS, every task is assigned a numeric priority at creation.

Higher numbers indicate higher priority.

The

scheduler always selects the highest-priority task in the Ready state. A lower-priority task only runs when all higher-

priority tasks are Blocked, Suspended, or Deleted.

Low & Idle Tasks

Priority 1/0 — run when

higher idle

High Priority Task

Priority 2 — preempts

lower tasks

Critical ISR Deferred

Priority 3 — top CPU

preference

Priority starvation is a real risk — if a high-priority task never blocks, lower-priority tasks are permanently denied CPU

time. Good RTOS design ensures tasks block on queues, semaphores, or delays to relinquish control periodically.

Two-Task System

The canonical FreeRTOS priority experiment creates exactly two tasks — one at a higher priority, one at a lower priority —

and observes their execution interleaving via UART output or GPIO toggles.

Task A — High Priority (2)

Prints a message repeatedly. Because it holds the higher

priority, it dominates CPU time. It must call

vTaskDelay()

to

allow Task B any runtime.

Task B — Low Priority (1)

Prints an alternating message. Executes only during the

window when Task A is in the Blocked state waiting for its

delay period to expire.

Both tasks are created before

vTaskStartScheduler()

is called. The scheduler then takes full control of execution

order based solely on priority and task state.

Working Mechanism

Understanding the state machine governing each FreeRTOS task is essential to predicting execution behaviour. Tasks

transition between four core states driven by API calls and scheduler decisions.

The critical path for our two-task experiment: Task A runs

→

calls

vTaskDelay()

→

moves to Blocked

→

Task B promoted to

Running

→

Task A delay expires

→

Task B preempted

→

Task A resumes Running.

Implementation Steps

Follow these steps to reproduce the priority experiment on any FreeRTOS-supported target (STM32, ESP32, Arduino

with FreeRTOS port, etc.).

1

Configure FreeRTOSConfig.h

Set

configUSE_PREEMPTION 1

, define

configTICK_RATE_HZ

(1000 recommended), and set

configMAX_PRIORITIES

to at least 5.

2

Create Task Functions

Write

vTaskA()

and

vTaskB()

as infinite loops. Include a

vTaskDelay(pdMS_TO_TICKS(500))

inside Task A to yield

periodically.

3

Register Tasks via xTaskCreate()

Call

xTaskCreate(vTaskA, "TaskA", 128, NULL, 2, NULL)

and

xTaskCreate(vTaskB, "TaskB", 128, NULL, 1, NULL)

before

starting the scheduler.

4

Start the Scheduler

Call

vTaskStartScheduler()

. Control never returns to

main()

. Monitor output via UART or a logic analyser on

a GPIO toggle pin.

Stack size (128 words here) must be sufficient for each task's local variables and function call depth. Increase if

stack overflow faults occur.

Output Behaviour

With preemption enabled and Task A calling

vTaskDelay(500 ms)

, the UART terminal produces a predictable, alternating

pattern that confirms the scheduler is operating correctly.

Expected UART Output

Task

A

:

Running

(High Priority)

Task

B

:

Running

(Low Priority)

Task

B

:

Running

(Low Priority)

Task

B

:

Running

(Low Priority)

Task

A

:

Running

(High Priority)

Task

B

:

Running

(Low Priority)

...

Task B may execute multiple iterations during Task A's 500 ms

block window depending on Task B's own loop timing and delay

configuration.

Timing Breakdown

:

0

–500 ms

Task A executes, then blocks

:

–500 ms

0

Task B runs during A's block

:

ms

500

Task A wakes, immediately preempts

B

Repeats

every scheduler tick cycle

Observations

Running this experiment yields several foundational insights about FreeRTOS scheduling that apply to all real-world

embedded designs.

Priority Is Absolute (Not Time-Sliced)

Without

vTaskDelay()

in Task A, Task B receives zero

CPU time. Priority preemption is not weighted — it is

binary: highest ready task runs exclusively.

Blocking Is the Yield Mechanism

The only way a high-priority task yields CPU is by

entering the Blocked state via delays, semaphores,

queues, or event groups. Busy-wait loops starve the

system.

Determinism Is Preserved

The output pattern is completely reproducible across

runs. This determinism is the defining characteristic

of an RTOS and distinguishes it from a general-

purpose OS.

Idle Task Confirms Correct

Configuration

If the Idle task runs (observable via hook), it confirms

no runnable user tasks are present — a useful sanity

check during bringup and debugging sessions.

Conclusion

Task priority in FreeRTOS is the primary lever for controlling execution order and guaranteeing real-time

responsiveness. This experiment confirms that the preemptive scheduler is strict, deterministic, and fully governed by

priority assignment and task state transitions.

Assign Priorities

Deliberately

Match priority levels to timing

criticality — ISR-deferred tasks

highest, background housekeeping

lowest.

Always Block, Never Busy-

Wait

Use

vTaskDelay()

, semaphores, or

queues to yield CPU. Tight loops

will starve all lower-priority tasks

completely.

Validate With Real Output

Use UART logs or GPIO toggles

with a logic analyser to empirically

verify scheduling behaviour during

development and integration

testing.

Mastering FreeRTOS task priority is foundational to building reliable, deterministic firmware for any embedded

real-time application.