Experiment 6: FreeRTOS Task

Implementation

A hands-on exploration of real-time operating system concepts using FreeRTOS on STM32, demonstrating task

scheduling, LED control, and efficient CPU utilisation through CMSIS-RTOS v2.

EMBEDDED SYSTEMS

FREERTOS

STM32

Aim

This experiment focuses on two core objectives that introduce the foundational concepts of real-time task

management on embedded hardware.

Implement FreeRTOS Task

Create and register a FreeRTOS task using the

CMSIS-RTOS v2 API on an STM32 microcontroller,

understanding task structure and lifecycle.

Control LED Using Scheduler

Demonstrate the FreeRTOS scheduler's role by

toggling an onboard LED at precise intervals,

validating real-time task execution behaviour.

Introduction

The Super Loop Problem

Traditional embedded firmware relies on a bare-metal

super loop — a single

while(1)

that executes tasks

sequentially. This approach blocks execution, wastes

CPU cycles during delays, and scales poorly with

complexity.

RTOS to the Rescue

A Real-Time Operating System replaces the super loop

with a

preemptive scheduler

. Each functional unit

becomes an independent task. The scheduler decides

which task runs, for how long, and when to yield —

enabling concurrent-like execution on a single core.

Theory

Understanding FreeRTOS requires familiarity with three core concepts that govern how tasks are created, managed, and

executed.

1

Task Stack

Every FreeRTOS task is allocated

its

own private stack

in RAM.

Local variables, function call

frames, and CPU register context

are stored here. Stack size must

be carefully configured to avoid

overflow.

2

Scheduler

The FreeRTOS scheduler uses a

priority-based preemptive

algorithm. It context-switches

between tasks, saving and

restoring CPU state. The highest-

priority ready task always runs

first.

3

CMSIS-RTOS v2

CMSIS-RTOS v2 is a standardised

API layer over FreeRTOS. It

abstracts vendor-specific RTOS

calls, making code portable

across ARM Cortex-M devices

and compatible with

STM32CubeIDE middleware.

Key Concepts

Creating a Task —

os

T

hre

adN

ew()

Tasks are registered with the RTOS kernel using

osThreadNew()

.

This function accepts a pointer to the task function, an optional

argument, and a set of thread attributes (name, stack size,

priority).

osThreadNew

LedTask, NULL, &led_attr);

(

Once created, the task enters the

Ready

state and waits for the

scheduler to give it CPU time.

Yielding with

os

D

el

a

y()

Inside a task, calling

osDelay(ms)

places the task in the

Blocked

state for the specified duration. This immediately hands the

CPU back to the scheduler, allowing other tasks or the idle task

to run — a key advantage over a busy-wait delay.

Task States

Running

— Currently executing

Ready

— Waiting for CPU

Blocked

— Waiting on delay/event

Suspended

— Explicitly paused

How It Works

Scheduler

Resume

Call osDelay

Toggle LED

The LED blink task operates in an infinite loop. On each iteration, it toggles the LED GPIO pin and then calls

osDelay(500)

.

During the 500 ms blocked period, the CPU is free — the FreeRTOS idle task runs, or other higher-priority tasks can

execute. This is fundamentally more efficient than a blocking

HAL_Delay()

in a super loop.

Implementation Steps

The experiment is set up using STM32CubeIDE with the following configuration pipeline.

GPIO Configuration

Configure the onboard LED pin

(

e.g., PA5 on Nucleo-F446RE) as

GPIO Output

in STM32CubeMX.

Set default output level to Low.

Enable FreeRTOS

Middleware

In CubeMX, navigate to

Middleware

→

FreeRTOS

and

enable it with CMSIS-RTOS v2

interface. Set the timebase source

to a timer other than SysTick (e.g.,

TIM1).

Create Task

Add a new task in the FreeRTOS

configuration tab. Generate code,

then implement the task function

body — toggle LED and call

osDelay(500)

inside

while(1)

.

Debugging with SWV ITM Console

SWV ITM Data Console

Serial Wire Viewer (SWV) with ITM

(

Instrumentation Trace Macrocell) provides a low

-

overhead

printf-style debug channel

over the

SWD interface — no UART required.

Enable SWV in the debug configuration and set

the core clock frequency correctly to receive

output.

Implementing

_write()

To redirect

printf

output to the ITM console, override the

_write()

syscall in your project:

int

_write(

int

file, char *ptr,

int

len

)

{

for

(

int

i =

0

; i <

len

; i++)

ITM_SendChar(*ptr++);

return

len

;

}

Once configured,

printf("Task running\n")

inside the FreeRTOS

task will print timestamped output to the SWV ITM Data

Console in STM32CubeIDE.

Observations

LED Blink Behaviour

The onboard LED toggled at a consistent

500

ms

interval

, producing a visible 1 Hz blink pattern. The

timing was accurate and stable, confirming correct

FreeRTOS scheduler operation and

osDelay()

resolution.

ITM Console Output

The SWV ITM Data Console displayed repeated print

messages (e.g.,

"LED Task Running"

)

at each task

activation cycle. This confirmed that the task was

being scheduled, executed, and released correctly by

the FreeRTOS kernel without any missed executions

or crashes.

Conclusion

This experiment successfully demonstrated the fundamentals of FreeRTOS task management on an STM32

microcontroller, highlighting the practical advantages of an RTOS over traditional bare-metal approaches.

RTOS Enables

Multitasking

Independent tasks can execute

concurrently, making complex

embedded applications

manageable and modular.

Efficient CPU Usage

osDelay()

blocks the task and frees

the CPU, unlike

HAL_Delay()

which

wastes cycles in a busy wait.

Superior to Super Loop

RTOS-based design scales

gracefully — adding new tasks

does not break existing timing

behaviour, unlike a monolithic

super loop.

FreeRTOS with CMSIS-RTOS v2 provides a portable, industry-standard foundation for building reliable, real-

time embedded applications on ARM Cortex-M devices.