Introduction to GPIO with STM32

This experiment focuses on configuring and controlling

GPIO (General Purpose Input/Output)

pins using an STM32

microcontroller. GPIO pins act as the primary interface between the microcontroller and external hardware

components, making them the fundamental building block of embedded systems development.

In this practical, we control a simple LED connected to the

STM32 Nucleo-F446RE

board. By writing a basic

embedded C program and using STM32 development tools, we make the LED blink at a fixed interval 4 a classic "Hello,

World!" for embedded engineers.

This experiment serves as a foundation for embedded systems development, as GPIO operations are essential in

almost every hardware-based application, from blinking an LED to driving complex motor controllers.

EMBEDDED SYSTEMS LAB

STM32

GPIO

Objectives

By the end of this experiment, students will be able to interface simple hardware components like LEDs using software

control. The following five objectives guide the learning journey, taking you from conceptual understanding to hands-

on implementation with industry-standard tools.

1

Understand GPIO Pins

Learn the concept of GPIO pins in STM32 microcontrollers and how they serve as the bridge between

software and hardware.

2

Configure with CubeMX

Learn how to configure GPIO pins using STM32CubeMX, a graphical tool that simplifies peripheral setup

and code generation.

3

Write LED Blinking Code

Write a program for LED blinking using HAL libraries, gaining practical experience with embedded C

programming.

4

Master Timing Control

Understand timing control using software delays and observe how delay values affect the blinking rate.

5

Use STM32CubeIDE

Gain hands-on experience with STM32CubeIDE, the integrated development environment used for coding,

compiling, and debugging.

Apparatus and Requirements

The following components and tools are required to perform this experiment. Each item plays a specific role in the

development and testing process.

Hardware Required

STM32 Nucleo-F446RE

— Development board with

onboard LED on pin PA5

USB Cable

(Type A to Mini B) — For powering the

board and uploading code

Computer/Laptop

— To run the development tools

Software Required

STM32CubeIDE

— Integrated development

environment for coding, compiling, and debugging

embedded applications

STM32CubeMX

— Graphical configuration tool to set

up peripherals and generate initialization code

automatically

Both STM32CubeIDE and STM32CubeMX are

free downloads from STMicroelectronics.

Understanding GPIO

GPIO

stands for

General Purpose Input/Output

. These pins allow the microcontroller to interact with the outside

world — reading signals from sensors or sending control signals to actuators. GPIO pins are the most basic yet most

frequently used peripheral in any embedded system.

Input Mode

Input pins read signals from

external components such as

sensors, switches, or buttons.

The microcontroller samples

the voltage level on the pin

and interprets it as a logic

HIGH or LOW.

Output Mode

Output pins control external

devices like LEDs, motors, or

displays. In this experiment,

we configure a GPIO pin as an

output to drive the onboard

LED.

Alternate Functions

GPIO pins in STM32 are highly

flexible and can also support

multiple alternate functions

like communication protocols

(

UART, SPI, I2C), making

them versatile for complex

designs.

GPIO in STM32 Microcontrollers

In STM32 microcontrollers, GPIO pins are organized into

ports

— GPIOA, GPIOB, GPIOC, and so on. Each port contains

up to 16 pins (P0 to P15). For this experiment, we use

Pin PA5

, which is internally connected to the onboard LED on the

Nucleo board.

GPIO Configuration Parameters

Pin Mode

Select Input or Output direction

Output Type

Push-Pull or Open-Drain

Speed Setting

Low, Medium, High, or Very High

Pull-up / Pull-down

Internal resistor configuration

Our Configuration: PA5

For the LED blinking experiment, PA5 is configured as:

Mode:

GPIO Output

Type:

Push-Pull

Speed:

Low Speed

Pull-up/down:

None

These settings ensure correct electrical behavior of the

pin and safe operation of the onboard LED.

Register Programming vs HAL Libraries

There are two main approaches to control GPIO in STM32 microcontrollers. Each has its own advantages depending on

the project requirements and the developer's experience level.

In this experiment, we use

HAL (Hardware Abstraction Layer) libraries

to simplify the implementation. HAL

functions wrap the low-level register operations into easy-to-use API calls, allowing beginners to focus on application

logic rather than hardware details.

Development Workflow

The complete workflow of this experiment follows a structured process that ensures smooth integration between

hardware configuration and software development. Each step builds on the previous one, guiding you from initial setup

to a working LED blink program.

01

Open STM32CubeMX

Launch the graphical configuration tool to begin setting

up the microcontroller.

02

Select the Board

Choose STM32 Nucleo-F446RE from the board selector.

03

Configure PA5 as GPIO Output

Set pin PA5 to GPIO_Output mode with Push-Pull output

type.

04

Generate Initialization Code

Let CubeMX generate the project files and HAL

initialization code.

05

Open Project in STM32CubeIDE

Import the generated project into the IDE for coding.

06

Write LED Blinking Logic

Add HAL_GPIO_TogglePin() and HAL_Delay() calls in the

main loop.

07

Compile and Upload

Build the project and flash the firmware onto the Nucleo

board.

08

Observe LED Behavior

Verify that the onboard LED blinks at the expected

interval.

This workflow ensures smooth integration between configuration and coding, reducing manual setup errors and

accelerating development time.

Program Explanation

The LED blinking program is based on a simple

infinite loop

that continuously toggles the LED state with a fixed delay

between each toggle. This is the embedded equivalent of "Hello, World!"

Core Logic

Toggle the LED state (ON

→

OFF or OFF

→

1.

ON)

2.

Wait for a fixed delay (500 ms)

Repeat continuously inside

while(1)

3.

Key HAL Functions

HAL_GPIO_Togg

lePin()

Changes the state of

the specified pin — if

the LED is ON, it turns

OFF, and vice versa.

HAL_Delay()

Pauses program

execution for the

specified time in

milliseconds using the

SysTick timer.

Working Sequence:

LED turns ON

→

Wait 500 ms

→

LED turns OFF

→

Wait 500 ms

→

Repeat. This creates a

continuous blinking effect with a 1-second full cycle.

Results and Observations

After successfully compiling and uploading the program to the STM32 Nucleo board, the following results were

observed, confirming correct hardware and software integration.

Pin Configured

GPIO pin PA5 was configured

correctly as a digital output in

Push-Pull mode.

LED Blinking

The onboard LED started blinking

continuously at the programmed

interval.

Delay Verified

Different delay values were

tested and the blinking rate

changed accordingly.

Observation: Delay vs Blink Rate

Increasing Delay

³

Slower Blinking

Longer HAL_Delay() values result in a slower blink

frequency.

Decreasing Delay

³

Faster Blinking

Shorter HAL_Delay() values produce a faster blink

frequency.

'	D	ó

# Conclusion and Learning Outcomes

This experiment successfully demonstrated how to configure and control GPIO pins in STM32 microcontrollers. By using STM32CubeMX and HAL libraries, the complexity of embedded programming was significantly reduced, making it accessible for beginners.

## GPIO Configuration	CubeMX & CubeIDE Workflow

Understood how to configure GPIO pins for 	Gained familiarity with the complete development output mode and control hardware directly 	workflow from graphical configuration to code through software.	deployment.

## HAL Function Usage	Timing-Based Operations

Learned to use HAL library functions like 	Implemented software delays to control real-time HAL_GPIO_TogglePin() and HAL_Delay() for 	behavior, forming a strong foundation for hardware control.	advanced embedded applications.

Mastering GPIO is the first step toward building complex embedded systems — from simple LEDs to full sensor networks and motor controllers.